Register Number: _____________________

DHANALAKSHMI SRINIVASAN ENGINEERING COLLEGE, PERAMBALUR – 621 212

(AUTONOMOUS)

DEPARTMENT OF AERONAUTICAL ENGINEERING

U20AE502/ Aircraft Structures

ASSESSMENT TEST – I

Year /Semester: III / V						Date:

Duration: 90 minitues						Total Marks: 60

Part A: 10 X 2 = 20 Marks

Answer all the questions

| 1 | Write the bending equation? | 2 | CO1 | BTL2 |
| --- | --- | --- | --- | --- |
| 2 | Why are the semimonocoque structures used? | 2 | CO1 | BTL2 |
| 3 | Define Unsymmetrical bending | 2 | CO1 | BTL2 |
| 4 | Explain how unsymmetrical bending is developed in a beam. | 2 | CO1 | BTL2 |
| 5 | Define neutral axis. | 2 | CO1 | BTL2 |
| 6 | Write the assumptions of plate theory. | 2 | CO2 | BTL2 |
| 7 | Define Shear Centre and Shear Flow. | 2 | CO2 | BTL2 |
| 8 | Define stress resultants. | 2 | CO2 | BTL2 |
| 9 | Write the formulas for moment of inertia? | 2 | CO2 | BTL2 |
| 10 | Define principal planes and principal stresses. | 2 | CO2 | BTL2 |

Part B: 2 X 13=26; 1X14=14=40 Marks

Answer all the questions

| 11 | a | i | Derive an expression for the bending stress of a body due to unsymmetric bending. | 13 | CO1 | BTL3 |
| --- | --- | --- | --- | --- | --- | --- |
|  | or |  |  |  |  |  |
|  | b | ii | A beam having the cross-section shown in Fig. is subjected to a bending moment of 1500 N-m in a vertical plane. Calculate the maximum direct stress due to bending stating the point at which it acts. | 13 | CO1 | BTL4 |
| 12 | a | i | Find the normal stress distribution of a thin-walled Z-section, thickness t, height of the web h, and flange width h/2, when it's subjected to a bending moment Mx. | 13 | CO1 | BTL4 |
|  | or |  |  |  |  |  |
|  | b | ii | A bending moment of 3000 Nm is applied to the purlin in a plane at an angle 30° to vertical axis. If the sense of bending moment is such that component Mx and My both produces tension in positive XY quadrant, calculate the maximum direct stress in purlin stating clearly at which point it acts. | 13 | CO1 | BTL4 |
| 13 | a | i | Find the bending stress at location A and B of I-section and calculate the angle. A 10 kN·m bending moment acts at an angle of 30° as shown in the figure. | 14 | CO2 | BTL3 |
|  | or |  |  |  |  |  |
|  | b | ii | Derive an expression for determining the shear stress of an open section beam with a neat sketch. | 14 | CO2 | BTL3 |

Prepared by	Verified by