DHANALAKSHMI SRINIVASAN ENGINEERING COLLEGE

THURAIYUR ROAD, PERAMBALUR – 621107

DEPARTMENT OF AERONAUTICAL ENGINEERING

PR8451- MECHANICS OF MACHINES

INTERNAL ASSESSMENT TEST- II

YEAR/SEM: II/ IV                                                                                                                         MAX.MARKS: 60

TIME: 1.30Hrs

PART – A (10X02=20)

Answer all the questions

- What is greasy friction?

- State the laws of dry friction?

- What is angle of repose?

- Define Co-efficient of friction

- Why self-locking screws have lesser efficiency?

- Write the importance of balancing?

- Write different types of balancing?

- Whether grinding wheels are balanced or not? If so why?

- Differentiate between the unbalanced force caused due to rotating and reciprocating masses?

- Define tractive force?

Part B (2 x 13 = 26marks)

- A) A vertical screw with single start square threads 50 mm mean diameter and 12.5 mm pitch is raised against a load of 10 kN by means of a hand wheel, the boss of which is threaded to act as a nut. The axial load is taken up by a thrust collar which supports the wheel boss and has a mean diameter of 60 mm. If the coefficient of friction is 0.15 for the screw and 0.18 for the collar and the tangential force applied by each hand to the wheel is 100 N ; find suitable diameter of the hand wheel

OR

B) The pitch of 50 mm mean diameter threaded screw of a screw jack is 12.5 mm. The coefficient of friction between the screw and the nut is 0.13. Determine the torque required on the screw to raise a load of 25 kN, assuming the load to rotate with the screw. Determine the ratio of the torque required to raise the load to the torque required to lower the load and also the efficiency of the machine.

- A) An inside cylinder locomotive has its cylinder centre lines 0.7 m apart and has a stroke of

0.6 m. The rotating masses per cylinder are equivalent to 150 kg at the crank pin, and the reciprocating masses per cylinder to 180 kg. The wheel centre lines are 1.5 m apart. The cranks are at right angles. The whole of the rotating and 2/3 of the reciprocating masses are to be balanced by masses placed at a radius of 0.6 m. Find the magnitude and direction of the balancing masses. Find the fluctuation in rail pressure under one wheel, variation of tractive effort and the magnitude of swaying couple at a crank speed of 300 r.p.m.

OR

B) A four-cylinder vertical engine has cranks 150 mm long. The planes of rotation of the first,

second and fourth cranks are 400 mm, 200 mm and 200 mm respectively from the third crank

and their reciprocating masses are 50 kg, 60 kg and 50 kg respectively. Find the mass of the

reciprocating parts for the third cylinder and the relative angular positions of the cranks in order

that the engine may be in complete primary balance.

Part C (1 x 14 = 14marks)

- A) An electric motor driven power screw moves a nut in a horizontal plane against a force of 75 kN at a speed of 300 mm/min. The screw has a single square thread of 6 mm pitch on a major diameter of 40 mm. The coefficient of friction at the screw threads is 0.1. Estimate power of the motor.

OR

B) The following data apply to an outside cylinder uncoupled locomotive

Mass of rotating parts per cylinder = 360 kg ;

Mass of reciprocating parts per cylinder= 300 kg ;

Angle between cranks = 90° ;

Crank radius = 0.3 m ;

Cylinder centres = 1.75 m ;

Radius of balance masses = 0.75 m ;

Wheel centres = 1.45 m.

If whole of the rotating and two-thirds of reciprocating parts are to be balanced in planes of the driving wheels, find :

  - Magnitude and angular positions of balance masses,

  - Speed in kilometres per hour at which the wheel will lift off the rails when the load on each driving wheel is 30 kN and the diameter of tread of driving wheels is 1.8 m, and

  - Swaying couple at speed arrived at in (2) above.