UNIT V RAMJET PROPULSION

The engine specifications identify important aspects that govern the design of individual

components. The specifications include:

1. low manufacturing cost

2. compact

3. safe

4. durable (2000 hours or 500 cycles between major overhaul)

5. maintainable

Figure 4.1: 1-MW marine gas turbine schematic.

6. operates on No. 1 diesel fuel

7. acceptable efficiency at full load (0.55 lbm / Hp·hr)

8. acceptable efficiency at part load

9. emissions meet applicable requirements

10. potential use in power generation

The engine design point occurs at full load or 100 percent throttle. It is fixed by

Engine Cycle Calculations Steady-state cycle calculations were performed to obtain the combustor inlet and outlet conditions at both full load and idle. These calculations were performed using GasTurb 9, which is a commercially available gas turbine cycle program for simulating engines.

The engine was modeled using GasTurb’s two-spooled turboshaft engine configuration.

The design point was fixed with the inputs listed in Table 4.2 while the idle

point required the use of GasTurb’s off-design capabilities.

GasTurb’s off-design model uses generic compressor and turbine maps to predict

Engine design point. Setting Value Ambient Temperature 288 K Ambient Pressure 101.325 kPa

Inlet Flow Rate 5.5 kg/s

Pressure Loss Across Inlet 4 inches of H2O

Compressor Polytropic Efficiency 85%

Compressor Pressure Ratio 8

Bleed From Compressor 1.5% of mass flow

Burner Efficiency 99%

Maximum Pressure Loss Across Burner 5% of burner inlet

Turbine Inlet Temperature (TIT) 1200 K

Turbine Polytropic Efficiency 88% each

Mechanical Efficiency 99%

Pressure Loss Across the Exhaust 8 inches of H2O

Gearbox Efficiency 95%

GasTurb Settings Setting Value Total Temperature T1 288 K

Total Pressure P1 101.3 kPa Relative Humidity 0 % Inlet Corrected Flow Rate 5.5 kg/s

Intake Pressure Ratio 0.99

Pressure Ratio 8

Burner Exit Temperature 1200 K

Burner Design Efficiency 99.7 %

Burner Part-load Constant 1.6 (default value)

Fuel Heating Value 42.7392 MJ/kg

Water-to-fuel Ratio 0

Steam-to-fuel Ratio 0

Overboard Bleed 0.0825

Power Offtake 5 kW

HP Spool Mechanical Efficiency 94 %

Burner Pressure Ratio 0.95

Turbine Interduct Pressure Ratio 1

Turbine Exit Duct Pressure Ratio 1

Nozzle Pressure Ratio 1.0001

Nozzle Thrust Coefficient 0

LP Spool Mechanical Efficiency 99 %

Nominal PT Spool Speed 0 RPM

Compressor Polytropic Efficiency 85 %

HPT Polytropic Efficiency 88 %

PT Polytropic Efficiency 88 %

Engine operating parameters.

Station Full Load Idle

P (kPa) T (K) m˙ (kg/s) P (kPa) T (K) m˙ (kg/s)

1 101.3 288 5.5 101.3 288 2.2 2 100.3 288 5.5 100.3 288 2.2 3 802.3 574 5.4 260.4 414 2.1

4 762.2 1200 5.5 246.4 790 2.2 4.5 244.4 941 5.5 111.4 666 2.2

5 101.3 774 5.5 101.3 660 2.2 6 101.3 774 5.5 101.3 660 2.2

engine performance at low power settings. A particular operating point is found by

setting limiters that restrict the maximum permissible power output and iterating

over the cycle until these parameters reach their maximum value. In this case, idle

was found by limiting the TIT until the net power output by the cycle was closest to

zero.

The results from the GasTurb analysis are tabulated below. Table 4.3 displays

the temperature and pressure at each location in the engine while Table 4.4 lists the

fuel flow rate required by the engine.

Design Choices

Many decisions were made to perform the preliminary design. These decisions were

based on the engine specifications, existing designs, and recommendations published

in the literature. Combustion Efficiency It was reasonable to expect combustion efficiencies greater than 99 percent. Thus, c = 99.7%

Required fuel flow rate.

m˙ f (kg/s) Design 0.09421 Idle 0.02131 Pressure Loss Coefficient

The combustor was designed to utilize the maximum permissible pressure loss,

P3−4 P3 = 5% (4.3.2) Pressure Loss Factor

A slightly higher pressure loss factor than typical aircraft can combustors was specified.

turn before it enters the premixer.P3−4q3= 40 (4.3.3)

Inlet and Outlet Area

The combustor inlet and outlet are sized to match the compressor exit and turbine

inlet, respectively.A3 = 0.0093 m2 (4.3.4)A4 = 0.0161 m2 (4.3.5)Fuel System

The injectors must be capable of providing extremely small droplets. Droplets any

larger than 20 microns lead to excessive evaporation times and result in a lengthy

premixer. Thus,SMD = 20 μ m (4.3.6)109

High fuel line pressures are required for nozzles to produce such small droplets.

The high pressures will also reduce the number of pressure nozzles required to supply

the desired mass flow rate. A pressure differential across the nozzle of 500 psi was

chosen.

Pf = 3.4 MPa (4.3.7)At high pressures, nozzles typically spray with wide cone angles.

n = 90 
SwirlersThe hub at the centre of the two co-axial swirlers was sized so that a fuel nozzle maybe mounted inside.Dhub,1 = 20.0 mm A slightly larger value was used for the combustor swirler hub.Dhub,2 = 50.0 mm

All swirlers are made up of flat vanes with a thickness of 1.5 mm.

tv = 1.5mm

Liner to Casing Area Ratio The liner-to-casing area ratio was selected based on consideration of the annulus flowand liner heat transfer.

karea =AlinerAref= 0.85 Liner Wall The liner is to be fabricated from the nickel-based alloy Hastelloy X. The emissivity (Omega, 1998) and conductivity: (Haynes International, 2005) for this material are: "liner = 0.85 liner = 26 W/ m K (

Premixer Design

Premixers play an important role in modern combustors. Premixers are devices composed of one or more swirlers designed to mix the fuel and air prior to combustion, as shown in Figure 2.10. The performance of these devices is quantified by the mixedness or the homogeneity of the discharged mixture. The detrimental effects of unmixedness

on NOx emissions have been investigated by Fric (1992).

Design must also ensure that the fuel/air mixture does not reside in the premixer

for too long and autoignite. The mixture must also move fast enough to ensure that

flashback does not occur (Poeschl et al. , 1994). Autoignition is the spontaneous

ignition of a fuel/air mixture after a certain time lapse above the autoignition temperature.

Flashback occurs when the flame propagates along boundary layers or slow

moving flows to ignite the incoming fuel/air mixture.

Premixer concept.

Many premixer designs exist and various configurations have been investigated

by Li & Gutmark (2004). Joshi et al. (1994) documented the design of a fuel air

premixer for General Electric’s aero-derivative dry low NOx combustors whereas Lin

et al. (2004) experimented with a multi-holed concept.

2.6 Combustion Chemistry

Combustion is an exothermic reaction in which fuel and oxidizer, air in most cases,

react to release heat. Stoichiometric combustion occurs when the exact quantity of

oxidizer required to completely burn the fuel is supplied to the combustion reaction.

The mixture is said to be lean if more than the stoichiometric quantity of oxidizer is

supplied and rich if the amount of oxidizer is less than stoichiometric. The ratio of

this stoichiometric quantity of air to fuel mass is determined from an atom balance.

The stoichiometric relation for the combustion of a hydrocarbon fuel given by C_H_

in air is

C_H_ + a (O2 + 3.76N2) ! _CO2 +_2_H2O + 3.76aN2

26

where a = _ + _4 . Thus, the stoichiometric air-fuel ratio_AF_stoic=_m˙ am˙ f_stoic=

4.76a

where MWa and MWf are the molecular weights of fuel and air respectively.

The equivalence ratio _ is used to quantify whether the mixture is rich, lean, or

stoichiometric. It is defined as _ =􀀀AF_􀀀 stoic AF_act

The equivalence ratio is unity for a stoichiometric mixture. Values greater than unity

indicate that the mixture is fuel rich, whereas values less than unity indicate a lean

mixture.

Chemical Equilibrium

Equation  is an idealization as the products of combustion differ in high temperature

combustion. CO2, H2O, O2, N2, and other major species dissociate into minor

species such as H2, OH, CO, H, O, N, and NO. Calculation of the mole fractions of

these species may be performed using several approaches.

One approach, used by Olikara & Borman (1975) and discussed in detail by Turns

(2000), is called the equilibrium-constant approach. This approach is based on the

second law of thermodynamics, which states that the system will spontaneously shift

towards the point of maximum entropy. Once this point is reached, no more compositional

changes occur and the system is said to be in equilibrium. Thus, the

equilibrium temperature, pressure, and chemical composition may be determined for

a fixed internal energy, volume, and mass by applying the first and second laws of

thermodynamics with the equation of state.

2Chemical Kinetics

Chemical kinetics involves the study of chemical reactions and their rates. In many

combustion processes, the rate at which the chemical reaction proceeds towards equilibrium

controls the overall rate of combustion. In this case, other factors including

mixing and evaporation occur fast enough that they can be considered insignificant.

Chemical reaction rates also determine the speed of pollutants formation and destruction,

moreover, they are related to ignition and flame extinction. Lastly, the

minimum volume required for combustion is dictated by the chemical reaction rates

(Longwell & Weiss, 1955).

The global reaction of Equation 2.6.1 can be written as

F + a · Ox ! b · Pr

Equation 2.6.4 describes the reaction of 1 mole of fuel F with a moles of oxidizer

Ox to form b moles of product Pr. Experimental results show that the rate of fuel

consumption can be expressed by (Turns, 2000) d[XF ]dt= −kG(T)[XF ]n[XOx]m (2.6.5)

where [Xi] denotes the molar concentration of the ith species, kG is the global rate

coefficient, and the exponents n and m describe the reaction order. The constants

kG, n, and m for global reactions are derived from curvefitting of experimental data

and usually only hold their validity over a limited range of temperatures.

The global reaction of Equation 2.6.1 does not fully describe the complex combustion

process. The overall reaction of fuel and air consists of many sequential steps

involving intermediate species that are formed and destroyed. These individual reactions

that occur are called elementary reactions. A mechanism is the collection

of elementary reactions needed to describe the global reaction. Reaction mechanisms

may involve a few steps (as in H2-O2 combustion) or several hundred (as in

hydrocarbon-air combustion).

Reaction Mechanisms The oxidation of higher order paraffins has been studied by

many researchers. Subsequently, many empirical models have been derived consisting

of either a single global step or several quasi-global steps. Examples include the

important global reaction scheme for hydrocarbons of Westbrook & Dryer (1981) and

multi-step reaction schemes by Hautman et al. (1981) and Jones & Lindstedt (1988).

Constant pressure reactors.

A detailed 30-step reaction mechanism for ”jet fuel” was devised by Kollrack (1976).

Turns (2000) reviewed other important reaction mechanisms including the extended

Zeldovich mechanism for thermal NOx formation.

2.6.3 Combustion Modeling and Emissions Prediction

Many researchers have made improvements to the accuracy of emissions predictions

and combustor simulation over early empirical models by applying reactor theory. Examples

of early empirical models include those of Mellor (1976) and Lefebvre (1984).

Reactor theory couples chemical and thermal analysis to describe the detailed

evolution of a system using simplified thermodynamic models called reactors. Two

types of reactors are well-stirred reactors (WSR) and plug-flow reactors (PFR), illustrated

in  In a WSR, fuel and air are supplied at a steady rate and

instantly mix and form a homogeneous mixture. The mixture burns at a rate prescribed

by chemical kinetics and the products are expelled at the same rate. The

pressure, temperature, and species concentrations remain constant throughout the

reactor as a result of the perfectly mixed assumption. PFRs model one-dimensional

flow that reacts as it moves downstream. Here, mixing is ignored and the flow is

assumed perfectly mixed in the radial direction perpendicular to the flow.

The simple model of Rizk & Mongia (1995b), as shown in Figure 2.12, illustrates

the use of reactor theory to simulate gas turbine combustion. The model uses a

Combustor reactor model. Modified from Rizk & Mongia (1995b)

network of two reactors and is applicable to both conventional and modern combustors.

The WSR models combustion in the primary zone whereas a downstream PFR

models the reaction of any escaped fuel and dilution of combustion products. Others

have used more complex networks to improve the accuracy of these models by adding

reactors for the pilot, cooling slots, and other features of interest (Rizk & Mongia,

1993b,a, 1995a; Tonouchi et al. , 1998; Andreini & Facchini, 2002).

While capable of describing the overall combustion trends, the reactor models

discussed above cannot fully describe the complex processes such as swirl, recirculation,

fuel injection, atomization, fuel evaporation, mixing, convective and radiative

heat transfer. Empirical or semi-empirical models can be combined with analytical

CFD models to provide a qualitative understanding of these processes with reasonably

accurate quantitative results. The three-dimensional model of a gas turbine

combustor developed by Rizk & Mongia (1991) is one example. The model uses

three-dimensional finite-volume CFD to solve the flow-field and is followed by the

application of empirical correlations by Lefebvre (1984, 1985) to yield the quantities

of interest. Rodriguez & O’Brien (1999) describe an unsteady, finite-rate model that

combines one-dimensional finite-volume methods with the global reaction mechanism

of Westbrook & Dryer (1981). Figure 2.13 illustrates a CFD solution of a combustor

using finite-rate chemistry

Figure 2.13: Combustor temperature and NO distribution using CFD and finite-rate

Combustion Performance

Gas turbine combustors must operate with stability over a wide range of operating

conditions (temperature, pressure, velocity, etc.) while maintaining good combustion

efficiency. They must also be capable of igniting with ease and re-initiating

combustion during operation in the event of a flameout.

2.7.1 Combustion Efficiency

Combustors operating at low combustion efficiencies are regarded as products of poor

design (Lefebvre, 1999). These low efficiencies may be directly correlated to wasted

fuel and high pollutant emissions. Typically governmental regulations require that

combustors operate at efficiencies of 99 percent or greater.

Combustion efficiency is dependant on the rate at which heat is released by the

chemical reaction. It is the ratio of the theoretical mass flow rate of fuel divided by

the actual value necessary to provide the same heat release. The combustion efficiency

is defined as, _c =m˙ ftheom˙ fac

where m˙ f is the mass flow rate of fuel.

The efficiency is governed by the time it takes to complete evaporation, mixing,

and the chemical reaction. Only one of these processes typically governs the system

at a given time. Childs (1950) has provided empirical relations for the estimation

of combustion efficiency based on engine data correlations. These relations assume

that combustion is governed by chemical reaction whereas the empirical correlations

of Lefebvre (1985) assume that both chemical reaction and evaporation processes

control the system.

Mixer Tube

The mixer is designed to ensure that complete evaporation and mixing occurs at

all operating conditions without the occurrence of autoignition. For simplicity, it

is assumed that mixing occurs instantaneously and the length is based solely on

considerations for evaporation. This implies that evaporation occurs at a much slower

rate than mixing, which is a reasonable assumption considering the low temperature

and pressure inside the premixer.

3.6.3.1 Droplet Evaporation

The evaporation time for the liquid fuel is estimated by modeling the spray as a single

droplet having the same SMD as that of the spray. The droplet, injected into the

mixing tube, is tracked as it travels through the bulk flow and evaporates. The length

necessary for complete evaporation is denoted by the axial distance where the droplet

diameter is zero, as illustrated in Figure 3.12.

Figure 3.12: Injection of droplet into mixer tube.

The droplet evaporation model used is described in detail by Hallett (2003), Turns

(2000), Kuo (1986), and Lefebvre (1989). It requires several simplifying assumptions:

1. the droplet is perfectly spherical,

2. quasi-steady state evaporation,

3. the transport properties remain constant spatially,

4. constant pressure,

5. pure fuel,

6. no chemical reaction,

7. uniform droplet temperature, and

8. unity Lewis number for the vapour/air mixture surrounding the droplet.

Quasi-steady evaporation means that any changes in droplet temperature or size

cause the vapour field surrounding the droplet to adjust instantaneously. This allows

the transient effects in the transport equations to be neglected.

With these assumptions, the evaporation of a droplet is described simply by

dD dt= −2Gl(3.6.18)where D = the droplet diameter

G = the mass flux of fuel vapour from the droplet surface

l = the liquid fuel density

76 The mass flux G from the droplet surface, accounting for the effects of forced convection,

is

G = NuCpvD

ln (1 + BT ) (3.6.19)

where Nu = the droplet Nusselt number

= the mixture thermal conductivity

Cpf = the fuel vapour specific heat

BT = the Spalding Transfer number

The Spalding transfer number,

BT =YR − Y11 − YR(3.6.20)

where YR and Y1 denote the mass fraction of fuel vapour at the droplet surface and

in the surrounding atmosphere, respectively.

The Nusselt number is estimated using the Ranz-Marshall correlation for the

convective heat transfer to a sphere

Nu = 2􀀀1 + 0.3Re1/2Pr1/3(3.6.21)

where both the Prandtl number Pr and the Reynolds number Re are defined as

Pr =Cp(3.6.22)Re =VrD

where Vr is the relative velocity of the droplet. The mixture specific heat, viscosity,

and density are denoted by Cp, , and , respectively.

A heat balance is performed on the droplet surface assuming that the droplet

temperature is uniform throughout. The balance gives

qR = Ghfg +lCplD6dTldt

where qR = the heat flux to the droplet surface

hfg = the latent heat of vaporization of the fuel

Cpl = the specific heat of liquid fuel

Tl = the droplet temperature77Rearranging gives

dTldt=6lCplD(qR − Ghfg)

The heat transferred to the droplet surface is determined from the mass flux solution,

qR =GCpf (T1 − Tl)expGCpfD− 1(3.6.26)

where T1 is the temperature of the ambient surroundings.

The heat and mass transfer solutions are coupled with the vapour-liquid equilibrium

relationship at the liquid surface. Thus,1YR= 1 +P1Pv− 1MW1MWf

where the vapour pressure Pv is given by the Antoine equation and P1 is the pressure

of the surroundings. The molecular weight of the ambient surroundings (air) and the

fuel are denoted by MW1 and MWf , respectively.

Neglecting the variation of properties in the vapour field requires the use appropriate

mean values. Hubbard et al. (1975) states that a 1/3 rule should be used.

T =23Tl +13T1 Y =23YR +13Y1

The properties for the mixture are then evaluated at the mean temperature T and

fuel mass fraction Y . However, evaluating the properties of mixtures is inconvenient

for use in simple calculations. Hallett (2003) recommended evaluating the mixture

, , , and Cp as for air at T. In most cases Y is small enough that the mixture

comprises mostly air.