DHANALAKSHMI SRINIVASAN ENGINEERING COLLEGE (AUTONOMOUS)

PERAMBALUR - 621212

U23ECP51 –PROCESSORS AND CONTROLLERS LABORATORY

MANUAL

# PREPARED BY

# T.BOOPATHY, AP / ECE

## EXP. 1 BASIC ARITHMETIC USING 8086 PROGRAMMING

AIM:

To write an Assembly Language Program (ALP) for performing the Arithmetic and logical operation of two-byte numbers

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 01 | Microprocessor Kit | 8086 | 1 |
| 02 | Power Supply | +5 Vdc | 1 |

ALGORITHM:

- bit Addition

ARITHMETIC OPERATIONS

  - Initialize the MSBs of sum to 0

  - Get the first number.

  - Add the second number to the first number.

  - If there is any carry, increment MSBs of sum by 1.

  - Store LSBs of sum.

  - Store MSBs of sum.

- bit Subtraction

  - Initialize the MSBs of difference to Zero

  - Get the first number

  - Subtract the second number from the first number.

  - If there is any borrow, increment MSBs of difference by 1.

  - Store LSBs of difference

  - Store MSBs of difference.

- bit Multiplication

  - Get the multiplier.

  - Get the multiplicand

  - Initialize the product to 0.

  - Product = product + multiplicand

  - Decrement the multiplier by 1

  - If multiplicand is not equal to 0, repeat from step (d) otherwise store the product.

- bit Division

  - Get the dividend

  - Get the divisor

  - Initialize the quotient to 0.

  - Dividend = dividend – divisor

  - If the divisor is greater, store the quotient. Go to step g.

  - If dividend is greater, quotient = quotient + 1. Repeat from step (d) Store the dividend value as remainder.

## PROGRAM:

## ADDITION:

ARITHMETIC OPERATIONS

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 1000 | 8B 06 00 11 | MOV AX,[1100] | Get the first data in AX reg |
| 1004 | 03 06 02 11 | SUB AX,[1102] | Add data in 1102 from AX |
| 1008 | 89 06 04 11 | MOV[1104],AX | Move the result from AX to 1104 |
| 100C | F4 | HLT | Stop the program |

SUBTRACTION:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 1000 | 8B 06 00 11 | MOV AX,[1100] | Get the first data in AX reg |
| 1004 | 2B 06 02 11 | SUB AX,[1102] | Subtract data in 1102 from AX |
| 1008 | 89 06 04 11 | MOV[1104],AX | Move the result from AX to 1104 |
| 100C | F4 | HLT | Stop the program |

MULTIPLICATION:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 1000 | 8B 06 00 11 | MOV AX,[1100] | Move the data in 1100 to accumulator |
| 1004 | 8B 1E 02 11 | MOV BX, [1102] | Move the data in 1102 to BX register |
| 1008 | F7 E3 | MUL BX | Multiply the data in BX with AX |
| 100A | 89 60 04 11 | MOV [1104],AX | Store the lower order product |
| 100E | 89 16 05 11 | MOV [1105],DX | Store the higher order product |
| 1012 | F4 | HLT | Stop the program |

DIVISION:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C7 C0 08 00 | MOV AX, 0008 | Move the data to accumulator |
| 1004 | C7 C3 04 00 | MOV BX, 0006 | Move the data to BX register |
| 1008 | F7 F3 | DIV BX | Divide the dividend by divisor |
| 100A | 89 06 00 12 | MOV [1200], AX | Store the lower order product |
| 100E | 89 06 01 12 | MOV [1201], DX | Store the higher order product |
| 1012 | F4 | HLT | Stop the program |

OUTPUT:

ADDITION:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1100 | 02 | 1104 | 05 |
| 1101 | 05 | 1105 | 09 |
| 1102 | 03 |  |  |
| 1103 | 04 |  |  |

SUBTRACTION:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1100 | 03 | 1104 | 05 |
| 1101 | 05 | 1105 | 09 |
| 1102 | 01 |  |  |
| 1103 | 04 |  |  |

MULTIPLICATION:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1100 | 01 | 1104 | 03 |
| 1101 | 02 | 1105 | 08 |
| 1102 | 03 |  |  |
| 1103 | 04 |  |  |

DIVISION:

| Output address | Result |
| --- | --- |
| 1200 | 01 |
| 1201 | 02 |

## RESULT:

Thus, Arithmetic operations of two-byte numbers are performed and the result is stored.

EXP. 2

SORTING AND SEARCHING

SORTING

## AIM:

To write an Assembly Language Program (ALP) to sort a given array in ascending

and descending order

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 1. | Microprocessor kit | 8086 | 1 |
| 2. | Power Supply | +5 V dc | 1 |

ALGORITHM:

- Sorting in ascending order:

  - Load the array count in two registers C1 and C2.

  - Get the first two numbers.

  - Compare the numbers and exchange if necessary, so that the two numbers are in ascending order.

  - Decrement C2.

  - Get the third number from the array and repeat the process until C2 is 0.

  - Decrement C1 and repeat the process until C1 is 0.

## Sorting in descending order:

  - Load the array count in two registers C1 and C2.

  - Get the first two numbers.

  - Compare the numbers and exchange if necessary, so that the two numbers are in descending order.

  - Decrement C2.

  - Get the third number from the array and repeat the process until C2 is 0.

  - Decrement C1 and repeat the process until C1 is 0.

## PROGRAM: ASCENDING:

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 |  | 8A 0C | MOV CL,[SI] | Number of comparisons in CL |
| 1002 | L4 | C7 C6 00 12 | MOV SI,1200H | Initialize memory location for array size |
| 1006 |  | 8A 14 | MOV DL,[SI] | Get the count in DL |
| 1008 |  | 46 | INC SI | Go to next memory location |
| 1009 |  | 8A 04 | MOV AL,[SI] | Get the first data in AL |

| 100B | L3 | 46 | INC SI | Go to next memory location |
| --- | --- | --- | --- | --- |
| 100C |  | 8A 1C | MOV BL,[SI] | Get the second data in BL |
| 100E |  | 38 D8 | CMP AL,BL | Compare two data’s |
| 1010 |  | 72 08 | JNB L1 | If AL < BL go to L1 |
| 1012 |  | 4E | DEC SI | Else, Decrement the memory location |
| 1013 |  | 88 04 | MOV [SI],AL | Store the smallest data |
| 1015 |  | 88 D8 | MOV AL, BL | Get the next data AL |
| 1017 |  | E9 03 00 | JMP L2 | Jump to L2 |
| 101A | L1 | 4E | DEC SI | Decrement the memory location |
| 101B |  | 88 10 | MOV [SI],BL | Store the greatest data in memory location |
| 101D | L2 | 46 | INC SI | Go to next memory location |
| 101E |  | FE CA | DEC DL | Decrement the count |
| 1020 |  | 75 E9 | JNZ L3 | Jump to L3, if the count is not reached zero |
| 1022 |  | 88 04 | MOV [SI],AL | Store data in memory location |
| 1024 |  | FE C9 | DEC CL | Decrement the count |
| 1026 |  | 75 D4 | JNZ L4 | Jump to L4, if the count is not reached zero |
| 1028 |  | F4 | HLT | Stop |

DESCENDING:

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 |  | 8A 0C | MOV CL,[SI] | Number of comparisons in CL |
| 1002 | L4 | C7 C6 00 12 | MOV SI,1200H | Initialize memory location for array size |
| 1006 |  | 8A 14 | MOV DL,[SI] | Get the count in DL |
| 1008 |  | 46 | INC SI | Go to next memory location |
| 1009 |  | 8A 04 | MOV AL,[SI] | Get the first data in AL |
| 100B | L3 | 46 | INC SI | Go to next memory location |
| 100C |  | 8A 1C | MOV BL,[SI] | Get the second data in BL |
| 100E |  | 38 D8 | CMP AL,BL | Compare two data’s |
| 1010 |  | 72 08 | JB L1 | If AL < BL go to L1 |
| 1012 |  | 4E | DEC SI | Else, Decrement the memory location |
| 1013 |  | 88 04 | MOV [SI],AL | Store the smallest data |
| 1015 |  | 88 D8 | MOV AL, BL | Get the next data AL |
| 1017 |  | E9 03 00 | JMP L2 | Jump to L2 |
| 101A | L1 | 4E | DEC SI | Decrement the memory location |
| 101B |  | 88 10 | MOV [SI],BL | Store the greatest data in memory location |
| 101D | L2 | 46 | INC SI | Go to next memory location |
| 101E |  | FE CA | DEC DL | Decrement the count |
| 1020 |  | 75 E9 | JNZ L3 | Jump to L3, if the count is not reached zero |
| 1022 |  | 88 04 | MOV [SI],AL | Store data in memory location |
| 1024 |  | FE C9 | DEC CL | Decrement the count |
| 1026 |  | 75 D4 | JNZ L4 | Jump to L4, if the count is not reached zero |
| 1028 |  | F4 | HLT | Stop |

OUTPUT:

ASCENDING

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1200 | 04 | 1300 | 04 |
| 1201 | 5 | 1301 | 5 |
| 1202 | 7 | 1302 | 6 |
| 1203 | 6 | 1303 | 7 |
| 1204 | 8 | 1304 | 8 |

DESCENDING

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1200 | 04 | 1200 | 04 |
| 1201 | 5 | 1201 | 8 |
| 1202 | 7 | 1202 | 7 |
| 1203 | 6 | 1203 | 6 |
| 1204 | 8 | 1204 | 5 |

RESULT:

Thus, given array of numbers are sorted in ascending & descending order.

SEARCHING

## AIM:

To write an Assembly Language Program (ALP) to find the largest and smallest number in a given array

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 1. | Microprocessor kit | 8086 | 1 |
| 2. | Power Supply | +5 V dc | 1 |

ALGORITHM:

- Finding largest number:

  - Load the array count in a register C1.

  - Get the first two numbers.

  - Compare the numbers and exchange if the number is small.

  - Get the third number from the array and repeat the process until C1 is 0.

## Finding smallest number:

  - Load the array count in a register C1.

  - Get the first two numbers.

  - Compare the numbers and exchange if the number is large.

  - Get the third number from the array and repeat the process until C1 is 0.

## PROGRAM:

LARGEST

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 |  | C7 C6 00 12 | MOV SI,1200H | Initialize array size |
| 1004 |  | 8A 0C | MOV CL,[SI] | Initialize the count |
| 1006 |  | 46 | INC SI | Go to next memory location |
| 1007 |  | 8A 04 | MOV AL,[SI] | Move the first data in AL |
| 1009 |  | FE C9 | DEC CL | Reduce the count |
| 100B | L2 | 46 | INC SI | Move the SI pointer to next data |
| 100C |  | 3A 04 | CMP AL,[SI] | Compare two data’s |
| 100E |  | 73 02 | JNB 1012 | If AL > [SI] then go to L1 (no swap) |
| 1010 |  | 8A 04 | MOV AL,[SI] | Else move the large number to AL |
| 1012 | L1 | FE C9 | DEC CL | Decrement the count |
| 1014 |  | 75 F5 | JNZ L2 | If count is not zero go to L2 |
| 1016 |  | C7 C7 00 13 | MOV DI,1300H | Initialize DI with 1300H |
| 101A |  | 88 05 | MOV [DI],AL | Else store the biggest number in1300 location |
| 101C |  | F4 | HLT | Stop |

SMALLEST:

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 |  | C7 C6 00 12 | MOV SI,1200H | Initialize array size |
| 1004 |  | 8A 0C | MOV CL,[SI] | Initialize the count |
| 1006 |  | 46 | INC SI | Go to next memory location |
| 1007 |  | 8A 04 | MOV AL,[SI] | Move the first data in AL |
| 1009 |  | FE C9 | DEC CL | Reduce the count |
| 100B | L2 | 46 | INC SI | Move the SI pointer to next data |
| 100C |  | 3A 04 | CMP AL,[SI] | Compare two data’s |
| 100E |  | 73 02 | JB L1 | If AL < [SI] then go to L1 (no swap) |
| 1010 |  | 8A 04 | MOV AL,[SI] | Else move the large number to AL |
| 1012 | L1 | FE C9 | DEC CL | Decrement the count |
| 1014 |  | 75 F5 | JNZ L2 | If count is not zero go to L2 |
| 1016 |  | C7 C7 00 13 | MOV DI,1300H | Initialize DI with 1300H |
| 101A |  | 88 05 | MOV [DI],AL | Else store the biggest number in 1300 location |
| 101C |  | F4 | HLT | Stop |

OUTPUT:

LARGEST:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1200 | 04 | 1300 | 04 |
| 1201 | 01 |  |  |
| 1202 | 02 |  |  |
| 1203 | 03 |  |  |
| 1204 | 04 |  |  |

SMALLEST:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1200 | 04 | 1300 | 01 |
| 1201 | 01 |  |  |
| 1202 | 02 |  |  |
| 1203 | 03 |  |  |
| 1204 | 04 |  |  |

RESULT:

Thus, largest and smallest number is found in a given array.

EXP 3

COPYING A STRING

AIM:

To move a string of length FF from source to destination

## ALGORITHM:

- Initialize the data segment. (DS)

- Initialize the extra data segment. (ES)

- Initialize the start of string in the DS. (SI)

- Initialize the start of string in the ES. (DI)

- Move the length of the string (FF) in CX register.

- Move the byte from DS TO ES, till CX=0.

## PROGRAM:

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 |  | C7 C6 00 12 | MOV SI, 1200H | Initialize destination address |
| 1004 |  | C7 C7 00 13 | MOV DI, 1300H | Initialize starting address |
| 1008 |  | C7 C1 06 00 | MOV CX, 0006H | Initialize array size |
| 100C |  | FC | CLD | Clear direction flag |
| 100D | L1 | F3 A4 | MOVSB | Copy the contents of source into destination until count reaches zero |
| 100E |  | E2 FD | LOOP L1 | ………………………. |
| 1010 |  | F4 | HLT | Stop |

OUTPUT:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1200 | 01 | 1300 | 01 |
| 1201 | 02 | 1301 | 02 |
| 1202 | 03 | 1302 | 03 |
| 1203 | 04 | 1303 | 04 |
| 1204 | 05 | 1304 | 05 |
| 1205 | 06 | 1305 | 06 |

RESULT:

Thus, a string of a particular length is moved from source segment to destination segment

## AIM:

SEARCHING A STRING

To scan for a given byte in the string and find the relative address of the byte from the

starting location of the string.

## ALGORITHM:

- Initialize the extra segment. (ES)

- Initialize the start of string in the ES. (DI)

- Move the number of elements in the string in CX register.

- Move the byte to be searched in the AL register.

- Scan for the byte in ES. If the byte is found ZF=0, move the address pointed by ES: DI to BX.

## PROGRAM:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C7 C6 00 13 | MOV SI,1300H | Initialize destination address |
| 1004 | C7 C7 00 14 | MOV DI, 1400H | Initialize starting address |
| 1008 | C7 C1 06 00 | MOV CX, 0006H | Initialize array size |
| 100C | FC | CLD | Clear direction flag |
| 100D | C6 C0 08 | MOV AL, 08H | Store the string to be searched |
| 1010 | F2 AE | REPNE SCASB | Scan until the string is found |
| 1012 | 4F | DEC DI | Decrement the destination address |
| 1013 | 8A 1D | MOV BL,[DI] | Store the contents into BL reg |
| 1015 | 88 1C | MOV [SI],BL | Store content of BL in source address |
| 1017 | F4 | HLT | Stop |

OUTPUT:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1300 | 01 | 1300 | 08 |
| 1301 | 02 |  |  |
| 1302 | 03 |  |  |
| 1303 | 04 |  |  |
| 1304 | 05 |  |  |
| 1305 | 08 |  |  |

RESULT:

Thus, a given byte or word in a string of a particular length in the extra segment (destination) is found.

## AIM:

FIND AND REPLACE

To find a character in the string and replace it with another character

## ALGORITHM:

- Initialize the extra segment. (E S)

- Initialize the start of string in the ES. (DI)

- Move the number of elements in the string in CX register.

- Move the byte to be searched in the AL register.

- Store the ASCII code of the character that has to replace the scanned byte in BL register.

- Scan for the byte in ES. If the byte is not found, ZF≠1 and repeat scanning. If the byte is found, ZF=1.Move the content of BL register to ES: DI

## PROGRAM:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C7 C7 00 13 | MOV DI,1300H | Initialize destination address |
| 1004 | C7 C6 00 14 | MOV SI,1400H | Initialize starting address |
| 1008 | C7 C1 06 00 | MOV CX, 0006H | Initialize array size |
| 100C | FC | CLD | Clear direction flag |
| 100D | C6 C0 08 | MOV AL, 08H | Store the string to be searched |
| 1010 | C6 C7 30 | MOV BH,30H | Store the string to be replaced |
| 1013 | F2 AE | REPNE SCASB | Scan until the string is found |
| 1015 | 4F | DEC DI | Decrement the destination address |
| 1016 | 8A 1D | MOV BL,[DI] | Store the contents into BL reg |
| 1018 | 88 1C | MOV [SI],BL | Store content of BL in source address |
| 101A | 88 3D | MOV [DI],BH | Replace the string |
| 101C | F4 | HLT | Stop |

OUTPUT:

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 1400 | 01 | 1300 | 01 |
| 1401 | 02 | 1301 | 02 |
| 1402 | 03 | 1302 | 03 |
| 1403 | 04 | 1303 | 04 |
| 1404 | 05 | 1304 | 05 |
| 1405 | 08 | 1305 | 30 |

RESULT:

Thus, a given byte or word in a string of a particular length in the extra segment (destination) is found and is replaced with another character.

## EXP. 5

## EXP. 8	STEPPER MOTOR INTERFACING

AIM:

To write an assembly language program in 8086 to rotate the motor at different speeds.

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 1. | Microprocessor kit | 8086 | 1 |
| 2. | Power Supply | +5V DC, +12 V DC | 1 |
| 3. | Stepper Motor Interface board | - | 1 |
| 4. | Stepper Motor | - | 1 |

ALGORITHM:

    - For running stepper motor clockwise and anticlockwise directions

    - Get the first data from the lookup table.

    - Initialize the counter and move data into accumulator.

    - Drive the stepper motor circuitry and introduce delay

    - Decrement the counter is not zero repeat from step(iii)

    - Repeat the above procedure both for backward and forward directions.

## SWITCHING SEQUENCE OF STEPPER MOTOR:

| MEMORY LOCATION | A1 | A2 | B1 | B2 | HEX CODE |
| --- | --- | --- | --- | --- | --- |
| 4500 | 1 | 0 | 0 | 0 | 09 H |
| 4501 | 0 | 1 | 0 | 1 | 05 H |
| 4502 | 0 | 1 | 1 | 0 | 06 H |
| 4503 | 1 | 0 | 1 | 0 | 0A H |

PROGRAM:

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 | START | C7 C7 00 12 | MOV DI, 1200H | Initialize memory location to store the array of number |
| 1004 |  | C7 C1 04 00 | MOV CX, 0004H | Initialize array size |
| 1008 | L1 | 8A 05 | MOV AL,[DI] | Copy the first data in AL |
| 100A |  | E6 E0 | OUT 0C0,AL | Send it through port address |
| 100C |  | C7 C2 10 10 | MOV DX, 1010H | Introduce delay |
| 1010 | L2 | 4A | DEC DX |  |
| 1011 |  | 75 FD | JNZ L2 |  |
| 1013 |  | 47 | INC DI | Go to next memory location |
| 1014 |  | E2 F2 | LOOP L1 | Loop until all the data’s have been sent |
| 1016 |  | E9 E7 EF | JMP START | Go to start location for continuous rotation |

OUTPUT:

| Input address | Data | Output | Input address | Data | Output |
| --- | --- | --- | --- | --- | --- |
| 1200 | 09 | Motor rotates in clockwise direction | 1200 | 06 | Motor rotates in anti clockwise direction |
| 1201 | 02 |  | 1201 | 0A |  |
| 1202 | 0A |  | 1202 | 02 |  |
| 1203 | 06 |  | 1203 | 09 |  |

## RESULT:

Thus, the assembly language program for rotating stepper motor in both clockwise and anticlockwise directions is written and verified.

## EXP. 9	DIGITAL CLOCK

AIM:

To display the digital clock specifically by displaying the hours, minutes and seconds

using 8086 kits.

## APPARATUS REQUIRED:

| S. No. | Item | Specification |
| --- | --- | --- |
| 1 | Microprocessor kit | 8086 |
| 2 | Power Supply | 5V |

PRELIMINARY SETTINGS

Org 1000h

Store time value in  memory location 1500-  Seconds

1501-   Minutes

1502- Hours

## ALGORITHM:

    - Take the microprocessor kit.

    - Display the digital clock by displaying the hours, minutes and seconds.

    - Execute the program.

    - Get the output.

## PROGRAM:

| 102D | 75DF |  | JNZ S2 |
| --- | --- | --- | --- |
| 102F | C7C60015 |  | MOV SI,1500H |
| 1033 | 8A04 |  | MOV AL,[SI] |
| 1035 | FEC0 |  | INC AL |
| 1037 | 8804 |  | MOV [SI],AL |
| 1039 | 80F83C” |  | CMP AL,3CH |
| 103C | 75C2 |  | JNZ START |
| 103E | C6C000 |  | MOV AL,00H |
| 1041 | 8804 |  | MOV [SI],AL |
| 1043 | 40 |  | INC SI |
| 1044 | 8A04 |  | MOV AL,[SI] |
| 1046 | FEC0 |  | INC AL |
| 1048 | 8804 |  | MOV [SI],AL |
| 104A | 80F83C. |  | CMP AL,3CH |
| 104D | 75B1 |  | JNZ START |
| 1%F | C6CX00 |  | MOV AL,0 |
| 1052 | 8804 |  | MOV [SI],AL |
| 1054 | 4s |  | INC SI |
| 1055 | 8A04 |  | MOV AL,[SIj |
| 1057 | FEC0 |  | INC AL |
| 1059 | 8804 |  | MOV [SI],AL |
| 105B | 8OF818 |  | CMP AL,18H |
| 105E | 75A0 |  | JNZ START |
| 1000 | C6C000 |  | MOV AL,0 |
| 10d3 | 8804 |  | MOV [SI],AL |
| 1065 | E998FF |  | JMP START |
| 1068 | €:6C406 | DISPLAY | MOV AH,06H |
| 106B | C7C20016 |  | MOV DX,1600H |
| 106F | C6C501 |  | MOV CH,01H |
| 1072 | C.6C100 |  | MOV CL,0H |
| 1073 | CD05 |  | INT 5 |
| 1077 | C3 |  | RET |
| 1078 | C7C60015 | CONVERT | MOV SI,1500H |
| 107C | C 7C308I6 |  | MOV BX,1608H |
| 1080 | C6C024 |  | MOV AL,24H |
| 1083 | 8807 |  | MOV [BX],AL |
|  | SE | CONDS |  |
| 1085 | 8A04 |  | MOV AL,[SIj |
| 1087 | C6t400 |  | MOV AH,0 |
| l08A | C6060A |  | MOV DH,0AH |
| 108D | F§Fs |  | DIV DH |
| 108F | 80C430 |  | ADD AH,30H |
| 1092 | 4B |  | DEC BX |
| 1093 | 8827 |  | MOV [BX],AH |
| 1095 | 4B |  | DEC BX |
| 1096 | 80C030 |  | ADD AL,30H |

OUTPUT:

hours: minutes: seconds -	:	:

RESULT

Thus, the digital clock program has been written and executed using 8086 microprocessor kit and the output of digital clock was displayed as [hours: minutes: seconds] successfully.

## EXP. 10	INTERFACING PRGRAMMABLE KEYBOARD AND

DISPLAY CONTROLLER- 8279

AIM:

To display the rolling message “ HAI “ in the display.

## APPARATUS REQUIRED:

8086 Microprocessor kit, Power supply, Interfacing board.

## ALGORITHM:

Display of rolling message “ HAI “

      - Initialize the counter

      - Set 8279 for 8-digit character display, right entry

      - Set 8279 for clearing the display

      - Write the command to display

      - Load the character into accumulator and display it

      - Introduce the delay

      - Repeat from step 1.

## PROGRAM TABLE:

| ADDRESS | LABEL | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- | --- |
| 1000 | START | C7 C6 00 12 | MOV SI,1200H | Initialize array |
| 1004 |  | C7 C1 0F 00 | MOV CX,000FH | Initialize array size |
| 1008 |  | C6 C0 10 | MOV AL,10 | Store the control word for display mode |
| 100B |  | E6 C2 | OUT C2,AL | Send through output port |
| 100D |  | C6 C0 CC | MOV AL,0CC | Store the control word to clear display |
| 1010 |  | E6 C2 | OUT C2,AL | Send through output port |
| 1012 |  | C6 C0 90 | MOV AL,90 | Store the control word to write display |
| 1015 |  | E6 C2 | OUT C2,AL | Send through output port |
| 1017 | L1 | 8A 04 | MOV AL,[SI] | Get the first data |
| 1019 |  | E6 C0 | OUT C0,AL | Send through output port |
| 101B |  | E8 06 00 | CALL DELAY | Give delay |
| 101E |  | 46 | INC SI | Go & get next data |
| 101F |  | E2 46 | LOOP L1 | Loop until all the data’s have been taken |

| 1021 |  | E9 DC FF | JMP START | Go to starting location |
| --- | --- | --- | --- | --- |
| 1024 | DELAY | C7 C2 FF A0 | MOV DX,0A0FFH | Store 16bit count value |
| 1028 | L2 | 4A | DEC DX | Decrement count value |
| 1029 |  | 75 FD | JNZ L2 | Loop until count values becomes zero |
| 102B |  | C3 | RET | Return to main program |

OUTPUT:

| Input address | Data | Output |
| --- | --- | --- |
| 1200 | FF |  |
| 1201 | 98 |  |
| 1202 | 88 |  |
| 1203 | 9F |  |
| 1204 | FF |  |

## RESULT:

Thus, the output for the interfacing programmable keyboard & display controller 8279 was executed successfully.

## EXP. 13	A/D AND D/A INTERFACE AND WAVEFORM GENERATION INTERFACING ANALOG – TO – DIGITAL CONVERTER

AIM:

To write an assembly language program to convert an analog signal into a digital signal using an ADC interfacing.

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | Microprocessor kit | 8086 | 1 |
| 2 | Power Supply | +5V DC, +12 V DC | 1 |
| 3 | ADC Interface board | - | 1 |

ALGORITHM:

  - Select the channel and latch the address.

  - Send the start conversion pulse.

  - Read EOC signal.

  - If EOC = 1 continue else go to step (iii)

  - Read the digital output.

  - Store it in a memory location.

## PROGRAM

| ADDRESS | OPCODE | PROGRAM | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C6 C0 00 | MOV AL,00 | Load accumulator with value for ALE high |
| 1003 | E6 C8 | OUT 0C8H,AL | Send through output port |
| 1005 | C6 C0 08 | MOV AL,08 | Load accumulator with value for ALE low |
| 1008 | E6 C8 | OUT 0C8H,AL | Send through output port |
| 100A | C6 C0 01 | MOV AL,01 | Store the value to make SOC high in the accumulator |
| 100D | E6 D0 | OUT 0D0H,AL | Send through output port |
| 100F | C6 C0 00 | MOV AL,00 | Introduce delay |
| 1012 | C6 C0 00 | MOV AL,00 |  |
| 1015 | C6 C0 00 | MOV AL,00 |  |
| 1018 | C6 C0 00 | MOV AL,00 | Store the value to make SOC low the accumulator |
| 101B | E6 D0 | OUT 0D0H,AL | Send through output port |

| 101D | E4 D8 | L1 : IN AL, 0D8H | Read the EOC signal from port & check for end of conversion |
| --- | --- | --- | --- |
| 101F | 80 E0 01 | AND AL,01 |  |
| 1022 | 80 F8 01 | CMP AL,01 |  |
| 1025 | 75 F6 | JNZ L1 | If the conversion is not yet completed, read EOC signal from port again |
| 1027 | E4 C0 | IN AL,0C0H | Read data from port |
| 1029 | C7 C3 00 11 | MOV BX,1100 | Initialize the memory location to store data |
| 102D | 88 07 | MOV[BX],AL | Store the data |
| 102F | F4 | HLT | Stop |

OUTPUT:

| Analog Data | Output address | Output |
| --- | --- | --- |
|  | 1100 | 0F |

RESULT:

Thus, the ADC was interfaced with 8086 and the given analog inputs were converted into its digital equivalent.

## INTERFACING DIGITAL – TO – ANALOG CONVERTER

AIM:

- To write an assembly language program for digital to analog conversion

- To convert digital inputs into analog outputs & to generate different waveforms

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | Microprocessor kit | 8086 | 1 |
| 2 | Power Supply | +5V DC, +12V DC | 1 |
| 3 | DAC Interface board | - | 1 |

ALGORITHM:

  - Measurement of analog voltage:

  - Send the digital value of DAC.

  - Read the corresponding analog value of its output.

## Waveform generation:

Square Waveform:

- Send low value (00) to the DAC.

- Introduce suitable delay.

- Send high value to DAC.

- Introduce delay.

- Repeat the above procedure. Saw-tooth waveform:

- Load low value (00) to accumulator.

- Send this value to DAC.

- Increment the accumulator.

- Repeat step (ii) and (iii) until accumulator value reaches FF.

- Repeat the above procedure from step 1. Triangular waveform:

- Load the low value (00) in accumulator.

- Send this accumulator content to DAC.

- Increment the accumulator.

- Repeat step 2 and 3 until the accumulator reaches FF, decrement the accumulator and send the accumulator contents to DAC.

- Decrementing and sending the accumulator contents to DAC.

- The above procedure is repeated from step (i)

## PROGRAM:

MEASUREMENT OF ANALOG VOLTAGE:

| PROGRAM | COMMENTS |
| --- | --- |
| MOV AL,7FH | Load digital value 00 in accumulator |
| OUT C0,AL | Send through output port |
| HLT | Stop |

PROGRAM TABLE:

Square Wave

| ADDRESS | OPCODE | PROGRAM | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C6 C0 00 | L2 : MOV AL,00H | Load 00 in accumulator |
| 1002 | E6 C0 | OUT C0,AL | Send through output port |
| 1004 | E8 12 10 | CALL L1 | Give a delay |
| 1007 | C6 C0 FF | MOV AL,FFH | Load FF in accumulator |
| 100A | E6 C0 | OUT C0,AL | Send through output port |
| 100C | E8 12 00 | CALL L1 | Give a delay |
| 1010 | 75 F9 | JMP L2 | Go to starting location |
| 1012 | C6 C0 05 | L1 : MOV CX,05FFH | Load count value in CX register |
| 1015 | FE C0 | L3 : LOOP L3 | Decrement until it reaches zero |
| 1017 | C3 | RET | Return to main program |

Saw tooth Wave

| ADDRESS | OPCODE | PROGRAM | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C6 C0 00 | L2 : MOV AL,00H | Load 00 in accumulator |
| 1003 | E6 C0 | L1 : OUT C0,AL | Send through output port |
| 1005 | FE C9 | INC AL | Increment contents of accumulator |
| 1007 | 75 F9 | JNZ L1 | Send through output port until it reaches FF |
| 1009 | 75 F9 | JMP L2 | Go to starting location |

Triangular Wave

| ADDRESS | OPCODE | PROGRAM | COMMENTS |
| --- | --- | --- | --- |
| 1000 | C6 C0 00 | L3 : MOV AL,00H | Load 00 in accumulator |
| 1003 | E6 C0 | L1 : OUT C0,AL | Send through output port |
| 1005 | FE C9 | INC AL | Increment contents of accumulator |
| 1007 | 75 F9 | JNZ L1 | Send through output port until it reaches FF |
| 1009 | C6 C0 FF | MOV AL,0FFH | Load FF in accumulator |
| 100C | E6 E0 | L2 : OUT C0,AL | Send through output port |
| 100E | FE C9 | DEC AL | Decrement contents of accumulator |
| 1010 | 8A 04 | JNZ L2 | Send through output port until it reaches 00 |
| 1013 | 75 F9 | JMP L3 | Go to starting location |

Result:

Thus, the DAC was interfaced with 8086 and different waveforms have been generated.

## EXP. 14	BASIC ARITHMETIC AND LOGICAL OPERATIONS

ARITHMETIC OPERATIONS

AIM:

To write a program to perform arithmetic operations of two 8-bit numbers using 8051 microcontroller.

## APPARATUS REQUIRED:

| S. No. | Item | Specification | Quantity |
| --- | --- | --- | --- |
| 01 | Microcontroller Kit | 8051 | 1 |
| 02 | Power Supply | +5 Vdc | 1 |

ALGORITHM:

  - Clear Program Status Word.

  - Select Register bank by giving proper values to RS1 & RS0 of PSW.

  - Load accumulator A with any desired 8-bit data.

  - Load the register R 0  with the second 8- bit data.

  - Add these two 8-bit numbers.

  - Store the result.

  - Stop the program.

## PROGRAM:

- 8 Bit Addition (Immediate Addressing)

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | CLR | C | C3 | Clear CY Flag |
| 4101 |  | MOV | A, # data 1 | 74,data1 | Get the data1 in Accumulator |
| 4103 |  | ADDC | A, # data 2 | 24,data2 | Add the data1 with data2 |
| 4105 |  | MOV | DPTR, # 4500H | 90,45,00 | Initialize the memory location |
| 4108 |  | MOVX | @ DPTR, A | F0 | Store the result in memory location |
| 4109 | L1 | SJMP | L1 | 80,FE | Stop the program |

- 8 Bit Subtraction (Immediate Addressing)

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | CLR | C | C3 | Clear CY flag |
| 4101 |  | MOV | A, # data1 | 74, data1 | Store data1 in accumulator |

| 4103 |  | SUBB | A, # data2 | 94,data2 | Subtract data2 from data1 |
| --- | --- | --- | --- | --- | --- |
| 4105 |  | MOV | DPTR, # 4500 | 90,45,00 | Initialize memory location |
| 4108 |  | MOVX | @ DPTR, A | F0 | Store the difference in memory location |
| 4109 | L1 | SJMP | L1 | 80,FE | Stop |

- 8 Bit Multiplication

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | MOV | A ,#data1 | 74, data1 | Store data1 in accumulator |
| 4102 |  | MOV | B, #data2 | 75,data2 | Store data2 in B reg |
| 4104 |  | MUL | AB | F5,F0 | Multiply both |
| 4106 |  | MOV | DPTR, # 4500H | 90,45,00 | Initialize memory location |
| 4109 |  | MOVX | @ DPTR, A | F0 | Store lower order result |
| 401A |  | INC | DPTR | A3 | Go to next memory location |
| 410B |  | MOV | A,B | E5,F0 | Store higher order result |
| 410D |  | MOVX | @ DPTR, A | F0 |  |
| 410E | STOP | SJMP | STOP | 80,FE | Stop |

- 8 Bit Division

| ADDRESS | LABEL | MNEMONIC | OPERAND | HEX CODE | COMMENTS |
| --- | --- | --- | --- | --- | --- |
| 4100 |  | MOV | A, # data1 | 74,data1 | Store data1 in accumulator |
| 4102 |  | MOV | B, # data2 | 75,data2 | Store data2 in B reg |
| 4104 |  | DIV | A,B | 84 | Divide |
| 4015 |  | MOV | DPTR, # 4500H | 90,45,00 | Initialize memory location |
| 4018 |  | MOVX | @ DPTR, A | F0 | Store remainder |
| 4109 |  | INC | DPTR | A3 | Go to next memory location |
| 410A |  | MOV | A,B | E5,F0 | Store quotient |
| 410C |  | MOV | @ DPTR, A | F0 |  |

| 410D | STOP | SJMP | STOP | 80,FE | Stop |
| --- | --- | --- | --- | --- | --- |

OUTPUT:

8051 - 8 BIT ADDITION

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 4102 | 04 | 4500 | 05 |
| 4104 | 01 |  |  |

8051 - 8 BIT SUBTRACTION

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 4102 | 04 | 4500 | 03 |
| 4104 | 01 |  |  |

8051 - 8 BIT MULTIPLICATION

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 4101 | 02 | 4500 | 06 |
| 4103 | 03 | 4501 | 00 |

8051 - 8 BIT DIVISION

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 4101 | 09 | 4500 | 03 |
| 4103 | 03 | 4501 | 00 |

## RESULT:

Thus, the 8051 ALP for arithmetic operation is executed.

## AIM:

LOGICAL AND BIT MANIPULATION

To write an ALP to perform logical and bit manipulation operations using 8051

microcontroller.

## ALGORITHM:

- Initialize content of accumulator as FFH

- Set carry flag (cy = 1).

- AND bit 7 of accumulator with cy and store PSW format.

- OR bit 6 of PSW and store the PSW format.

- Set bit 5 of SCON.

- Clear bit 1 of SCON.

- Move SCON.1 to carry register.

- Stop the execution of program.

## PROGRAM: OR:

| ADDRESS | LABEL | MNEMONICS | OPERAND | COMMENT |
| --- | --- | --- | --- | --- |
| 4100 | START: | MOV | A, #DATA1 | Copy to A Register first number |
| 4102 |  | ORL | A, #DATA2 | OR the A register with an immediate data 45h |
| 4104 |  | MOV | DPTR, #4500 | Store in Data Pointer the memory address |
| 4107 |  | MOVX | @DPTR, A | Store the result in Memory |
| 4109 | HERE: | SJMP | HERE | Halt here |

AND:

| ADDRESS | LABEL | MNEMONICS | OPERAND | COMMENT |
| --- | --- | --- | --- | --- |
| 4100 | START: | MOV | A, #DATA1 | Copy 1st Number to A register |
| 4102 |  | ANL | A, #DATA2 | AND A register with 2nd number |
| 4104 |  | MOV | DPTR, #4500 | Copy the memory address to Data Pointer |
| 4107 |  | MOVX | @DPTR, A | Store the result in Memory |
| 4108 | HERE: | SJMP | HERE | Halt here |

OUTPUT:

OR

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 4101 | 01 | 4500 | 04 |
| 4103 | 03 |  |  |

AND

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 4101 | 01 | 4500 | 03 |
| 4103 | 03 |  |  |

RESULT

Thus, the Logical and bit manipulation operation is done in 8051 microcontroller.

## EXP. 15	SQUARE AND CUBE PROGRAM, FIND 2’S COMPLEMENT OF A NUMBER

AIM:

To convert Square and Cube program, find 2’s complement of a number using 8051

microcontroller.

## APPARATUS REQUIRED:

- 8051 Microcontroller kit

- Keyboard

- Power supply

## ALGORITHM:

- Save contents of all register which are used in routine.

- Get the data.

- Perform 2’s complement using 8051 microcontroller.

- Restore contents of registers.

## PROGRAM TO FIND SQUARE:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 7500 | 90 80 00 | MOV DPTR,# 8000 | Initialize the memory pointer |
| 7503 | E0 | MOVX A,@DPTR | Get the number N |
| 7504 | F5 F0 | MOV B,A | Store it in B |
| 7506 | A4 | MUL AB | Square : N×N |
| 7507 | A3 | INC DPTR | Increment memory pointer |
| 7508 | F0 | MOVX @DPTR,A | Store lower byte of square |
| 7509 | A3 | INC DPTR | Increment memory pointer |
| 750A | E5 F0 | MOV A,B | Get higher byte of square |
| 750C | F0 | MOVX @DPTR,A | Store higher byte of square |
| 750D | 80 FE | SJMP 750D | Stop the program |

PROGRAM TO FIND CUBE:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 7500 | 90 80 00 | MOV DPTR,#8000H | Initialize the memory pointer |
| 7503 | E0 | MOVX A,@DPTR | Get the number N |
| 7504 | F8 | MOV R0,A | Store it in R0 |
| 7505 | F5 F0 | MOV B,A | Store it in B |
| 7507 | A4 | MUL AB | Square : N×N |
| 7508 | C0 F0 | PUSH B | Save the high byte of square |
| 750A | F5 F0 | MOV B,A | Save the lower byte of square in B |
| 750C | E8 | MOV A,R0 | Get the number N |
| 750D | A4 | MUL AB | N × lower byte of square of N |
| 750E | A3 | INC DPTR | Increment memory pointer |
| 750F | F0 | MOVX @DPTR,A | Save partial result 1 (lower byte) |
| 7510 | E5 F0 | MOV A,B | Get lower byte |
| 7512 | F9 | MOV R1,A | Save higher byte of N × lower byte of square of N (partial result 2) |
| 7513 | D0 F0 | POP B | Get the higher byte of square |
| 7515 | E8 | MOV A,R0 | Get the number N |
| 7516 | A4 | MUL AB | N × higher byte of square |
| 7517 | 29 | ADD A,R1 | Add to partial result 2 |
| 7518 | A3 | INC DPTR | Increment memory pointer |
| 7519 | F0 | MOVX @DPTR,A | Save middle byte |
| 751A | E5 F0 | MOV A,B | Get higher byte |
| 751C | 34 00 | ADDC A,#00 | Add carry of previous addition if any |
| 751E | A3 | INC DPTR | Increment memory pointer |
| 751F | F0 | MOVX @DPTR,A | Save higher byte |
| 7520 | 80 FE | SJMP | Stop the program |

PROGRAM TO FIND 2’s COMPLEMENT OF A NUMBER:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 7500 | 74 23 | MOVA, #23 | Move 23 to A register |
| 7502 | F4 | CPL A | Complement of A register |
| 7503 | 24 01 | ADD A,#01 | Add 1 to it to get 2’s complement |
| 7505 | 90 45 00 | MOV DPTR,#4500 | Initialize memory location |
| 7508 | F0 | MOVX @DPTR,A | Store the value in memory location |
| 7509 | 80 FE | SJMP | Stop the program |

OUTPUT:

PROGRAM TO FIND SQUARE

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 8000 | 02 | 8000 | 02 |
|  |  | 8001 | 04 |

PROGRAM TO FIND CUBE

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 8000 | 02 | 8000 | 02 |
|  |  | 8001 | 08 |

PROGRAM TO FIND 2’s COMPLEMENT OF A NUMBER

| Input address | Data | Output address | Result |
| --- | --- | --- | --- |
| 7501 | 02 | 4500 | FD |

RESULT:

Thus, the Square and Cube program, finding 2’s complement of a number is done in 8051 microcontroller and is executed successfully.

## EXP. 16	UNPACKED BCD TO ASCII AIM:

To convert BCD number into ASCII by using 8051 microcontroller

## APPARATUS REQUIRED:

- 8051 Microcontroller kit

- Keyboard

- Power supply

## ALGORITHM:

- Save contents of all registers which are used in routine.

- Get the data in AL register and make AH equal to 00.

- Use AAM instruction to convert number in its decimal equivalent in the unpacked format

- Add 30H in each digit to get its ASCII equivalent.

- Display digit one by one using function 2 if INT 21H.

- Restore contents of registers.

## PROGRAM:

| ADDRESS | OPCODE | MNEMONICS | COMMENTS |
| --- | --- | --- | --- |
| 4000 | 75 14 | MOV A,# 14 | Load the first data in A register |
| 4002 | E5 F0 | MOV B, A | Move reg A to reg B |
| 4004 | F3 0F FC | ANL A, #0FH | Mask the bit |
| 4006 | 24 30 | ADD A, #30H | Add 30H with reg A |
| 4008 | 90 45 00 | MOV DPTR,#4500 | Initialize the data pointer for storing the result |
| 400B | F0 | MOVX@DPTR,A | Result move to external memory |
| 400C | E5 F0 | MOV A, B | Transfer B register to A register |
| 400E | F3 F0 | ANL A, #F0H | Mask the bit |
| 4010 | E4 | SWAP A | Swap lower and upper nibble |
| 4011 | 24 30 | ADD A, #30H | Add 30H to reg A |
| 4013 | A3 | INC DPTR | Increment data pointer |
| 4014 | F0 | MOVX@DPTR,A | Move reg A value to data pointer |
| 4016 | 80 FE | SJMP | Stop the program |

RESULT:

The given BCD number is converted into ASCII using 8051 microcontroller kit.